STM32_SPI Communication test.

Project creation.☒

Pin Selection.☒

SPI2 selected as device.

PB12->SPI2_NSS

PB13->SPI2_SCK

PB14->SPI2_MISO,

PB15->SPI2_MOSI

Implement SPI Write/Read APIs.☒

STM32 + ENC28 communication.

ENC28J60 Initialization and Write/Read API development.☒

ENC28J60 Interrupt pin management related functions development.

Configure LWIP for SRM32.

Build the LWIP library with bare minimum configurations. ☒

Initialize the timer for tick.

Link the LWIP with ENC28J60 device using port files “ethernetif.c”.☐

Add the TCP/UDP support only.

Test Loopback mode if possible.

If Loopback not possible then Add dummy Tx/Rx frames and test the output.

Add dummy “ethernetif.c” and “ethernetif.h” file support and trace the calls to Tx/Rx functions.

Implement “ethernetif.c” and “ethernetif.h” files for the LWIP for compatibility with ENC28J60 controller.

Test UDP broadcasting and bug fix.

Test UDP Unicast and bug fix.

Test TCP Communication and bug fix.

Test HTTP Communication and bug fix.

To integrate the Microchip ENC28J60 Ethernet controller with the lwIP (Lightweight IP) stack on a microcontroller, you must interface your hardware SPI layer with lwIP's network interface layer (netif). [1, 2]

Below is a complete, minimal framework for running lwIP in a bare-metal (NO_SYS = 1) configuration. [1, 2]

Step 1: Configuration (lwipopts.h)

This configuration file tells lwIP to run without an operating system layer (polling mode). [1, 2]

# c

# #ifndef __LWIPOPTS_H__

# #define __LWIPOPTS_H__

# #define NO_SYS                  1  // 1 = Bare-metal (No OS / No FreeRTOS)

# #define SYS_LIGHTWEIGHT_PROT    0

# #define MEM_ALIGNMENT           4

# /* Memory optimizations for smaller microcontrollers */

# #define MEM_SIZE                2048

# #define MEMP_NUM_PBUF           8

# #define MEMP_NUM_TCP_PCB        4

# #define MEMP_NUM_UDP_PCB        4

# /* Enable UDP and TCP */

# #define LWIP_UDP                1

# #define LWIP_TCP                1

# #define LWIP_DHCP               0  // 0 = Static IP, 1 = DHCP

# #endif

# Use code with caution.

# Step 2: The Network Interface Port (ethernetif.c)

# This file links the low-level SPI functions of the ENC28J60 driver to the packet buffers (pbuf) of the lwIP stack. [1]

# c

# #include "lwip/netif.h"

# #include "lwip/pbuf.h"

# #include "netif/etharp.h"

# #include "enc28j60_driver.h" // Replace with your chip-specific SPI driver

# /* Low-level hardware initialization */

# static void low_level_init(struct netif *netif) {

# netif->hwaddr_len = ETHARP_HWADDR_LEN;

# // Set MAC Address (e.g., 00:08:DC:00:00:01)

# netif->hwaddr[0] = 0x00; netif->hwaddr[1] = 0x08;

# netif->hwaddr[2] = 0xDC; netif->hwaddr[3] = 0x00;

# netif->hwaddr[4] = 0x00; netif->hwaddr[5] = 0x01;

# netif->mtu = 1500;

# netif->flags = NETIF_FLAG_BROADCAST | NETIF_FLAG_ETHARP | NETIF_FLAG_LINK_UP;

# ENC28J60_Init(netif->hwaddr); // Call your SPI physical layer init code

# }

# /* Transmit packet over SPI to ENC28J60 */

# static err_t low_level_output(struct netif *netif, struct pbuf *p) {

# struct pbuf *q;

# ENC28J60_TransmitStart(p->tot_len); // Alert ENC28J60 of oncoming payload

# for(q = p; q != NULL; q = q->next) {

# // Send buffer chunk via SPI

# ENC28J60_WriteBuffer(q->payload, q->len);

# }

# ENC28J60_TransmitSend(); // Execute transmit command

# return ERR_OK;

# }

# /* Receive packet from ENC28J60 over SPI */

# static struct pbuf* low_level_input(struct netif *netif) {

# struct pbuf *p = NULL, *q;

# uint16_t len = ENC28J60_GetReceivedPacketLength();

# if (len == 0) return NULL;

# // Allocate packet buffer memory inside lwIP heap

# p = pbuf_alloc(PBUF_RAW, len, PBUF_POOL);

# if (p != NULL) {

# for (q = p; q != NULL; q = q->next) {

# ENC28J60_ReadBuffer(q->payload, q->len); // Read chunk via SPI

# }

# ENC28J60_FreeRxBuffer(); // Clear ENC28J60 RX memory slot

# }

# return p;

# }

# /* Initialization callback handled by lwIP */

# err_t enc28j60_netif_init(struct netif *netif) {

# netif->name[0] = 'e';

# netif->name[1] = 'n';

# netif->output = etharp_output; // Assigns the ARP handler

# netif->linkoutput = low_level_output;

# low_level_init(netif);

# return ERR_OK;

# }

# /* Poll this function inside your main application loop */

# void enc28j60_netif_poll(struct netif *netif) {

# struct pbuf *p = low_level_input(netif);

# if (p != NULL) {

# if (netif->input(p, netif) != ERR_OK) {

# pbuf_free(p);

# }

# }

# }

# Use code with caution.

# Step 3: Main Initialization and Runtime Loop (main.c)

# Initialize the stack, register your interface, configure static IP settings, and handle networking in the primary application loop. [1]

# c

# #include "lwip/init.h"

# #include "lwip/netif.h"

# #include "lwip/ip_addr.h"

# #include "lwip/timeouts.h"

# #include "ethernetif.h"

# struct netif enc28j60_netif;

# int main(void) {

# // 1. Initialize MCU clock, GPIO, and SPI peripherals here

# // 2. Initialize lwIP Stack

# lwip_init();

# // 3. Define Network Configurations (Static IP setup)

# ip4_addr_t ipaddr, netmask, gw;

# IP4_ADDR(&ipaddr, 192, 168, 1, 150);  // Static IP address

# IP4_ADDR(&netmask, 255, 255, 255, 0); // Netmask

# IP4_ADDR(&gw, 192, 168, 1, 1);        // Gateway IP

# // 4. Bind ENC28J60 interface to lwIP

# netif_add(&enc28j60_netif, &ipaddr, &netmask, &gw, NULL, &enc28j60_netif_init, &ethernet_input);

# netif_set_default(&enc28j60_netif);

# netif_set_up(&enc28j60_netif);

# // 5. Main Runtime Loop

# while (1) {

# // Look for inbound Ethernet data packets

# enc28j60_netif_poll(&enc28j60_netif);

# // Handle internal lwIP software timers (ARP, TCP retransmissions, etc.)

# sys_check_timeouts();

# }

# }

# Use code with caution.

Key Considerations

SPI Speed Constraints: The ENC28J60 clock maxes out at a 20 MHz SPI clock frequency. Ensure your microcontroller's SPI prescaler doesn't exceed this limit. [1, 2, 3, 4]

ERXWM Errata: The ENC28J60 contains silicon errata affecting packet reception filters. Verify that your low-level ENC28J60_Init code explicitly implements the official Microchip errata workarounds (such as enabling the EIE.PKTIE interrupt flag manually or applying custom receive buffer logic). [1]

Pre-made Driver Examples: If you are using popular hardware platforms, you can reference the Juddling Raspberry Pi Pico lwIP Port or the matwey generic C port for full implementations of the low-level SPI register maps. [1, 2]

If you want help writing or adapting the SPI driver code, let me know:

What microcontroller architecture you are using (e.g., STM32, AVR, RP2040)?

Whether you want to use Hardware Interrupts (INT pin) or Polling

//------------- Help ----------------//

Important registers

# ECON1 = 0x1F.

# ECON2 = 0x1E.

# 0x1F = Read ECON1.

# 0x5F = Write ECON1.

# 0x9F = BFS of ECON1.

# 0xBF = BFC of ECON1

# 0x1E = Read ECON2.

# 0x5E = Write ECON2.

# 0x9E = BFS of ECON2.

# 0xBE = BFC of ECON2

Sample codes.

# https://github.com/farhadkhodaei/smart-home-with-stm32f030f4p6-enc28j60

# https://github.com/STMicroelectronics/STM32CubeF4/tree/89e6d4466578bc9eab83de8fcd1e397ceb5e5cc9

# https://github.com/matwey/lwip-enc28j60/tree/e3efa062f9e988616d9a0e993eb65785cccc3b68

# https://github.com/Juddling/pi-pico-enc28j60/tree/10841d2ffceab00a773678c5da0570acbb49aae5

# https://github.com/njh/ethercard

# https://github.com/adamdunkels/uip (uIP stack)

--------------- Debugging needed ------------------

## LED configuration and ensure the data incoming and outgoing status.

## Print the Buffer memory registers while packet arrival to know the buffer points mismatch.

## Give separate power supply to MAC module.

## Read and set appropriately (EBSTCON).

## Confirm (After setting RXEN, the Duplex mode and the Receive Buffer Start and End Pointers should not be modified. Additionally, to prevent unexpected packets from arriving, it is  recommended that RXEN be cleared before altering the receive filter configuration (ERXFCON) and MAC address )

Debug flow.

PHCON1-> PLOOPBK(14) // Loopback mode on

bit 8 PDPXMD: PHY Duplex Mode bit(1)	// Check the duplex mode.

1 = PHY operates in Full-Duplex mode

0 = PHY operates in Half-Duplex mode

MACON1->

Full-Duplex config.

MACON3->FULDPX(0)

PHCON1->PDPXMD(8)

Pattern Match filter setting

Packats only from MAC “50:46:5D:4C:D6:D0” will be accepted.

# ERXFCON->PMEN = 1;

# ERXFCON->ANDOR = 1;

# ERXFCON-> CRCEN = 1;

EPMOH:EPMOL = 0000h

EPMM7:EPMM0 = 0000000000000FC0h

EPMCSH:EPMCSL = 7B9Ch