Please make me the configuration for the RCC to achieve HCLK = 32MHz and PCLK = 32 MHz.

Based on the STM32F0x reference manual (RM0091), here's the RCC configuration to achieve HCLK = 32 MHz and PCLK = 32 MHz using the PLL with HSE as the source.

## Configuration Steps

### 1. Clock Source: HSE (External Crystal)

- Assume HSE = 8 MHz (common crystal frequency).

- We will use the PLL to multiply it to 32 MHz.

### 2. PLL Configuration

- PLL Source: HSE

- PLL Multiplier: ×4 (8 MHz × 4 = 32 MHz)

- PLL Output: Must be ≤ 48 MHz (OK here).

### 3. System Clock Selection

- Select PLL as SYSCLK.

### 4. AHB & APB Prescalers

- To get HCLK = 32 MHz: AHB prescaler = 1 (no division).

- To get PCLK = 32 MHz: APB prescaler = 1 (no division).

## Register Settings

### RCC_CR (Clock Control Register)

- Enable HSE: HSEON = 1

- Wait for HSERDY = 1

### RCC_CFGR (Clock Configuration Register)

- Set PLL source = HSE: PLLSRC = 1 (HSE selected)

- Set PLL multiplier = ×4: PLLMUL[3:0] = 0010 (×4)

- Set AHB prescaler = /1: HPRE[3:0] = 0xxx (default = 0)

- Set APB prescaler = /1: PPRE[2:0] = 0xx (default = 0)

- Set system clock switch to PLL: SW[1:0] = 10

### RCC_CR

- Enable PLL: PLLON = 1

- Wait for PLLRDY = 1

### Switch to PLL

- After PLL is ready, set SW[1:0] = 10 (switch to PLL)

- Wait for SWS[1:0] = 10 (confirms PLL is system clock)

## Example Code (C - Bare Metal)

c

#include "stm32f0xx.h"

void SystemClock_Config(void) {

// 1. Enable HSE and wait for ready

RCC->CR |= RCC_CR_HSEON;                 // Enable HSE

while (!(RCC->CR & RCC_CR_HSERDY));      // Wait for HSE ready

// 2. Configure PLL: HSE as source, multiply by 4

RCC->CFGR &= ~RCC_CFGR_PLLSRC;           // Clear PLL source bits

RCC->CFGR |= RCC_CFGR_PLLSRC_HSE_PREDIV; // HSE as PLL source

RCC->CFGR &= ~RCC_CFGR_PLLMUL;           // Clear PLL multiplier bits

RCC->CFGR |= RCC_CFGR_PLLMUL4;           // PLL multiplier = 4 (8MHz * 4 = 32MHz)

// 3. Set AHB and APB prescalers to 1 (no division)

RCC->CFGR &= ~RCC_CFGR_HPRE;             // AHB prescaler = /1

RCC->CFGR &= ~RCC_CFGR_PPRE;             // APB prescaler = /1

// 4. Enable PLL and wait for ready

RCC->CR |= RCC_CR_PLLON;                 // Enable PLL

while (!(RCC->CR & RCC_CR_PLLRDY));      // Wait for PLL ready

// 5. Select PLL as system clock source

RCC->CFGR &= ~RCC_CFGR_SW;               // Clear system clock switch

RCC->CFGR |= RCC_CFGR_SW_PLL;            // Select PLL as system clock

while ((RCC->CFGR & RCC_CFGR_SWS) != RCC_CFGR_SWS_PLL); // Wait for switch

// Optional: Enable prefetch buffer and set flash latency for 32 MHz

FLASH->ACR |= FLASH_ACR_PRFTBE;          // Enable prefetch buffer

FLASH->ACR &= ~FLASH_ACR_LATENCY;        // Clear latency bits

FLASH->ACR |= FLASH_ACR_LATENCY_0;       // 1 wait state for 32 MHz

}

## Important Notes

- Flash Latency: At 32 MHz, you need 1 wait state (see Table 9 in the manual, page 75).

- HSE Value: If your HSE is not 8 MHz, adjust the PLL multiplier accordingly.

- PLL Input Range: Ensure HSE frequency is within 4–32 MHz.

- Clock Security System (CSS): Consider enabling CSS for reliability.

## Verification

- After configuration:

  - HCLK = 32 MHz

  - PCLK = 32 MHz

  - SYSCLK = 32 MHz (from PLL)

You can verify using the RCC->CFGR SWS bits and by toggling a GPIO at a known rate.

Let me know if you need the configuration for a different HSE frequency or using HSI as the source.

Please give me the process for HSI as main clock.

Here's the RCC configuration process using HSI (8 MHz internal RC) as the main clock source to achieve HCLK = 32 MHz and PCLK = 32 MHz.

## HSI-Based Configuration Process

### 1. Clock Source: HSI (Internal 8 MHz RC)

- HSI frequency: 8 MHz (factory calibrated to ±1%)

- We'll use the PLL to multiply it to 32 MHz

### 2. PLL Configuration

- PLL Source: HSI (8 MHz)

- PLL Multiplier: ×4 (8 MHz × 4 = 32 MHz)

- PLL Output: Must be ≤ 48 MHz (OK here)

### 3. System Clock Selection

- Select PLL as SYSCLK

### 4. AHB & APB Prescalers

- AHB prescaler = 1 (no division) → HCLK = 32 MHz

- APB prescaler = 1 (no division) → PCLK = 32 MHz

## Step-by-Step Process

### Step 1: Enable HSI

- Set HSION bit in RCC_CR

- Wait for HSIRDY flag to be set

### Step 2: Configure PLL

- Set PLLSRC[1:0] = 00 (HSI as PLL source)

- Set PLLMUL[3:0] = 0010 (×4 multiplier)

- Set AHB/APB prescalers = 1 (no division)

### Step 3: Enable PLL

- Set PLLON bit in RCC_CR

- Wait for PLLRDY flag to be set

### Step 4: Configure Flash Latency

- At 32 MHz, need 1 wait state (set LATENCY[2:0] = 001)

- Enable prefetch buffer (PRFTBE = 1)

### Step 5: Switch System Clock to PLL

- Set SW[1:0] = 10 (switch to PLL)

- Wait for SWS[1:0] = 10 (confirms PLL is system clock)

## Register Configuration Details

### RCC_CR (Clock Control Register) - Page 110

text

HSION = 1    // Enable HSI

Wait for HSIRDY = 1

PLLON = 1    // Enable PLL after configuration

Wait for PLLRDY = 1

### RCC_CFGR (Clock Configuration Register) - Page 111-113

text

PLLSRC[1:0] = 00    // HSI as PLL source

PLLMUL[3:0] = 0010  // PLL multiplication factor ×4

HPRE[3:0] = 0xxx    // AHB prescaler = /1 (default)

PPRE[2:0] = 0xx     // APB prescaler = /1 (default)

SW[1:0] = 10        // Switch to PLL after it's ready

### FLASH_ACR (Flash Access Control Register) - Page 69

text

LATENCY[2:0] = 001  // 1 wait state for 24-48 MHz

PRFTBE = 1          // Enable prefetch buffer

## Complete C Code Example

c

#include "stm32f0xx.h"

void SystemClock_Config_HSI(void) {

// 1. Enable HSI and wait for ready

RCC->CR |= RCC_CR_HSION;                 // Enable HSI

while (!(RCC->CR & RCC_CR_HSIRDY));      // Wait for HSI ready

// 2. Configure Flash latency for 32 MHz (1 wait state)

FLASH->ACR &= ~FLASH_ACR_LATENCY;        // Clear latency bits

FLASH->ACR |= FLASH_ACR_LATENCY_0;       // 1 wait state (24-48 MHz)

FLASH->ACR |= FLASH_ACR_PRFTBE;          // Enable prefetch buffer

// 3. Configure PLL: HSI as source, multiply by 4

RCC->CFGR &= ~RCC_CFGR_PLLSRC;           // Clear PLL source bits

RCC->CFGR |= RCC_CFGR_PLLSRC_HSI_PREDIV; // HSI as PLL source

RCC->CFGR &= ~RCC_CFGR_PLLMUL;           // Clear PLL multiplier bits

RCC->CFGR |= RCC_CFGR_PLLMUL4;           // PLL multiplier = 4 (8MHz * 4 = 32MHz)

// 4. Set AHB and APB prescalers to 1 (no division)

RCC->CFGR &= ~RCC_CFGR_HPRE;             // AHB prescaler = /1

RCC->CFGR &= ~RCC_CFGR_PPRE;             // APB prescaler = /1

// 5. Enable PLL and wait for ready

RCC->CR |= RCC_CR_PLLON;                 // Enable PLL

while (!(RCC->CR & RCC_CR_PLLRDY));      // Wait for PLL ready

// 6. Select PLL as system clock source

RCC->CFGR &= ~RCC_CFGR_SW;               // Clear system clock switch

RCC->CFGR |= RCC_CFGR_SW_PLL;            // Select PLL as system clock

while ((RCC->CFGR & RCC_CFGR_SWS) != RCC_CFGR_SWS_PLL); // Wait for switch

// Optional: Trim HSI for better accuracy if needed

// RCC->CR &= ~RCC_CR_HSITRIM;           // Clear trim bits

// RCC->CR |= (16 << 3);                 // Default trim value = 16

}

## HSI Calibration (Optional)

If you need better accuracy:

c

// Calibrate HSI against external reference (e.g., LSE via TIM14)

void HSI_Calibrate(void) {

// Use TIM14 input capture to measure HSI against LSE

// Adjust HSITRIM[4:0] in RCC_CR (bits 7:3)

// Each step ≈ 40 kHz at 8 MHz

// Example: increase frequency by 80 kHz

uint32_t trim = (RCC->CR & RCC_CR_HSITRIM) >> 3;

trim += 2;  // +2 steps ≈ +80 kHz

RCC->CR = (RCC->CR & ~RCC_CR_HSITRIM) | (trim << 3);

}

## Key Differences from HSE Configuration

- No external crystal needed - uses internal 8 MHz RC

- Lower accuracy (±1% factory, ±3% over temp/voltage)

- Faster startup - no crystal stabilization time

- Lower power - no external oscillator power

- Can be trimmed using HSITRIM bits for better accuracy

## Verification

After configuration:

- Check RCC->CFGR bits SWS[1:0] = 10 (PLL is system clock)

- HCLK = PCLK = 32 MHz

- SYSCLK = 32 MHz (from PLL)

Note: HSI is less accurate than HSE but useful when:

- No external crystal is available

- Low cost is critical

- Moderate timing accuracy is acceptable

- Fast startup is needed